Issue Report

Issue  		: Gflip 7 – Missing Receiver
Solution request	: Need visual inspection after assembly component 
Requestor 		: Fui Ling – TCL
Date   		: 1 Desember 2025

Model Gflip

Terdapat 3 model Gflip:

- Gflip 5

- Gflip 6

- Gflip 7

## Requirement Visual inspection

## Gflip 5

Current Process – Part yang dipasang dan perlu visual check oleh operator:

- Vibrator

  - Pemeriksaan: Missing component

- Rubber (Merah)

  - Pemeriksaan: Missing component, warna rubber, orientasi/reverse

- Receiver (Orientasi ke bawah sesuai panah)

  - Pemeriksaan: Missing component, orientasi receiver

- Antenna

  - Pemeriksaan: Missing component, komponen terangkat

- Magnet

  - Pemeriksaan: Missing component

## Gflip 6

Current Process – Part yang dipasang dan perlu visual check oleh operator:

- Vibrator

  - Pemeriksaan: Missing component

- Receiver (Orientasi ke atas sesuai panah)

  - Pemeriksaan: Missing component, orientasi receiver

- Antenna

  - Pemeriksaan: Missing component, komponen terangkat

- Magnet

  - Pemeriksaan: Missing component

- Dusket

  - Pemeriksaan: Missing component

## Gflip 7

Current Process – Part yang dipasang dan perlu visual check oleh operator:

- Vibrator

  - Pemeriksaan: Missing component

- Receiver (Orientasi ke bawah sesuai panah)

  - Pemeriksaan: Missing component, orientasi receiver

- Rubber (Black Color)

  - Pemeriksaan: Missing component, warna rubber, orientasi/reverse

- Magnet

  - Pemeriksaan: Missing component

- Dusket

  - Pemeriksaan: Missing component

- Antenna

  - Pemeriksaan: Missing component, komponen terangkat

## Environment (request to PID team / made by Engineer team)

- Box Cabinet

Sample :

    - Warna : Hitam

    - Bahan : akrilik

  - Ukuran: Persegi panjang (P x L x T – waiting research)

  - Sensor untuk deteksi unit (trigger start inspection)

  - Relay control untuk On/Off lighting

- Lighting

  - Qty: 1 Pcs

  - Bentuk: Square

  - Ukuran: (P x L – 400x200mm)

- Alarm / Speaker

  - Qty: 1 Pcs

- Mini PC / CPU

  - Qty: 1 Pcs

  - Minimal: Intel Core i7 (Gen 12)

  - RAM: Min. 8 GB DDR4

  - Storage: SSD 500 GB

  - OS: Windows 10

- Monitor

  - Qty: 1 Pcs

- Camera Hikrobot

  - Qty: 1 Pcs

  - Serial number: MV- CS200 -10UC

- Lens Hikrobot

  - Qty: 1 Pcs

  - Serial number: MVL-MF1228M-8MP

Total Investment (1 Unit inspection):

| No. | Item | Detail | Qty | Unit Price | Total Price |
| --- | --- | --- | --- | --- | --- |
| 1 | Box Cabinet |  | 1 Pcs | $500 | $500 |
| 2 | Lighting |  | 1 Pcs | $200 | $200 |
| 3 | Alarm |  | 1 Pcs | $60 | $60 |
| 4 | Mini PC / CPU |  | 1 Pcs | $500 | $500 |
| 5 | Monitor |  | 1 Pcs | $80 | $80 |
| 6 | Camera Hikrobot |  | 1 Pcs | $340 | $340 |
| 7 | Lens Hikrobot |  | 1 Pcs | $56 | $56 |
| Total |  |  |  |  | $1736 |